# OpenQSE System Architecture Session 02 Technical Transcript

Date: July 24, 2026 Original duration: 3h 33m 51s Sanitization note: Nontechnical setup, AV logistics, off-topic asides, and unrelated room conversation were removed. Technical discussion, technical questions, and working-group report-outs were retained.

## QEC Workflow and Architecture Driver

Room: And the point of this talk is going to be tying this quantum error correction a little bit to this general idea of workflows that we’ve been talking about. So my understanding is that some significant fraction of us probably don’t think about quantum error correction day-to-day. So, as we probably all know, quantum error correction provides a means to tolerate and ultimately overcome. that we, instead of utilizing a sort of a noisy physical qubit, we sort of take a whole bunch of noisy physical qubits and we make a logical qubit out of them. These logical qubits will still suffer errors, but now the information is spread in some non-local way. And so Even though the qubit, the individual physical qubits suffer errors, we can still hopefully detect them and then correct them.

Room: So typically with this quantum error correction codes, you sort of parameterize them by three different numbers, the number of physical qubits, logical qubits, and then the distance, which is basically a measure of how many errors you can effectively tolerate. And so, you know, some kind of high level, you sort of want not too many, and to be not too large, right? Because it’s a resource you have to provide. You want K to be large because you want to encode lots of logical qubits in there, and you want also D to be large because you want to tolerate more errors. They’re all sort of you know, related to each other, so there are limits on what you can do, but nevertheless, at a high level, that’s sort of what people aim for.

Room: But I don’t know if it’s a problem, maybe it’s not a problem, but it’s an interesting observation is that there’s no such thing as really best code for everyone. There are different hardware resources constraints, different noise types, gate locality. Some, you know, some logical in some codes, logical errors are easier than others. So, one example of this is, in fact, you know, if you consider, let’s say, circuits, and you can think about what Alice and Bob is doing versus what, let’s say, I own IBM or IQM are doing. The hardware, even though it’s still superconducting circuits, the hardware is susceptible to very different types of noise. So the error correcting codes that they will utilize will be very different. Therefore, the sort of underlying architecture will also be different. Different architectures, quantum error correction codes will have different specific requirements.

Room: So this is a super high level overview, a big picture of sort of how to implement a quantum error correcting code once again. So this is like a loop that you can think about running indefinitely for the lifetime of the experiment. So once again, you have a quantum computer. But so you have, you know, a whole bunch of physical qubits. You’ve sort of defined how you’re going to define your logical qubits among them. First thing you have to do, you have to make what we call a syndrome measurement. So that effectively means doing like a sort of a parity measurement. So you have to measure operators such as ZZZ or *** or some number of qubits. In most architectures, you cannot do this directly.

Room: So instead, we sort of run a circle like this, where we have some data qubits that are encoding your actual logical data, but then you introduce extra ancilla qubits, and then you perform these two qubit gates between them, and you measure the ancilla, and that gives you the sort of measurement that you’re after. So this syndrome tells you something about where errors have occurred. sort of, but then you have to pass them into some classical computing infrastructure. And that infrastructure is running a device called a decoder, syndrome decoder. So the decoder basically takes the syndrome data and then converts it into operation into sort of converts it into operations that will that will perform corrections, right? The syndrome tells you how to undo the errors that have happened. So for small error correcting codes, this is actually very simple.

Room: Sometimes you can have even a lookup table for larger as you sort of grow and you utilize more qubits, then these become more complicated and you actually have to do some sort of a computation. Okay, so that’s a classical computation that in general has to be done. And then the next step is applying the corrections and fixing the errors that have occurred. So I just want to highlight that everything I’ve talked about here right now, it’s really just sustaining this logical qubit. We’ve done these circuits, but the circuits are not doing any logical operations. They’re just making sure that the qubit stays alive. So if all you want to do that, then we sort of call this just logical memory.

Room: And it turns out that in this case, both correction and the decoding can actually be deferred to the end of the sort of quote un quote computation. You’re not really computing, but you’re just letting the qubit live for long. The qubit will lose its quantum nest, but as long as you reading these syndromes constantly, you can sort of correct everything at the end after you’ve read out the actual logical qubit. What we want to do is we want to do actual computations, again, at the logical level. So, you know, this is some circuit that we might want to run. Again, think of these qubits now as being logical qubits, each of them. corresponding to some number of physical qubits. So in practice, you then have to sort of fit these logical operations between the cycles of your actual quantum error correcting protocols, right?

Room: And now in order to then do this computation, you sort of have to define a logical universal gate set. So a number of logical gates that will let you implement completely arbitrary algorithms. So this is a little bit hairy, but like there are sort of you can divide them into two types. So with Clifford operations, both the corrections and the coding can actually be also deferred. With non-Clifford gates, you actually have to have done at the very least the real-time decoding. So before, so say you do a, you have a Clifford operation, non-Clifford operation that you want to do, you have to have sort of decoded up to that point.

Room: The correction is still optional, but there are some circumstances under which People still think about applying corrections directly, okay, but the point is that, because you now have to do, at the very least, this decoding, what that increases your sort of the constraints on the latency of the implementation. So, you know, what are the timing requirements associated with this? I’ll give you actual numbers for some particular experiment in a second, but here just highlight that, of course, that depends on the details of the architecture, quantum Mercury can code, and so on. But, you know, you just you want to do it fast enough so the errors don’t overwhelm your Computer. So this bit here, I can, there’s a paradigm by which I can basically delay this until the end. So I have to know what the, I have to basically do the decoding.

Room: Once I know the decoding, I sort of know how to apply. I meant for the T gate, not for the Cliffords. Yes, I even mean this is true for the T for the T gates. Yeah, so you have to, that’s why you have to have known, by knowing what the coding has done, by knowing what the correction is, you can basically choose. how you implement the T-gate. So this is so this is defined in the in this 2019 paper by Daniel, I forget the last name. I also didn’t know about this, so I sort of went on a bender looking at some number of papers, and I think this is in fact true.

Room: Now, there are cases where it’s not clear that this is actually what you would do, because once you have to put in the time to do the decoding, it may be that the, you know, applying the gates directly is true. Is just a small fraction of the classical compute, because you just have to generate your pulses and so on, so… Cubic coherence, you know, again, not too many, your qubits cannot be too noisy if the they’re too noisy, you have too many errors happening before you kind of can even read the syndrome out, then things are going to fail. You have to be able to do this gate without initialization fast enough, because you still have to run that syndrome extraction circuits.

Room: The classical processing, the coding, and then if you’re applying an active correction, pulse generation, that has to happen fast enough. and then you have to pass all the data around. So basically, to sort of figure out whether you’ve got all the right infrastructure in place, you have to take all of these things into the account. And so I would argue that sort of in contrast to the most application workflows that we discussed in the morning, one presumably needs somewhat more specialized hardware, right? At the very least for the decoding, I think this is widely believed now that people will need some sort of FPGAs or maybe GPUs that are Living very close to the hardware, so I wanted to, yep.

Room: The whole thing about the FPGAs and the GPUs, I mean, do you think that today’s GPUs or the current roadmap of GPUs are going to be able to handle this or are we kind of looking at like a ASICs GPU of some sort? I think so, I mean, ultimately, the type of the coding that you have to do depends sort of on the size of the… syndrome, the syndrome count that you have. So even today, and I will show you in my next slide, literally an example where using sort of off-the-shelf FPGA, the coding can be done in a fraction in 100 nanoseconds or something like this.

Room: Certainly the GPUs that you can get, you know, something like Frontier, that can do enormous amount of I would imagine that actually it’s a combination of things that one has to worry about, so both the time, the decoding time, but also some of the other sort of classical computer that is involved, and also the shuffling of the data around. It was more of a comment because I do assume this is a discussion session. I think GPUs are generally slow to decode. Right, they have the latency requirements are the latencies are very slow with them, and and. It depends whether you’re decoding surface codes or LDPC codes also. So I think the fastest ones I’ve seen is FPGAs, but also CPUs, surprisingly. So one is the latency associated with basically putting the data into the GPU versus the decoding capability of a GPU, right?

Room: Certainly if you ask NVIDIA, then there’s no issue with none of those things. So I think it’s still sort of, I mean, I think it’s not clear what people will end up utilizing down the road. But there are approaches where people are using GPUs, like NVIDIA, the recent NVIDIA paper from a couple of months ago. And there are cases which I’m going to talk about here where you just use standard FPGA. I sort of decided in the end, I’ll just look at whatever is maybe the latest. But what they do is they basically engineer the whole loop that I’ve described above, even including the active feedback, what they call just this active correction. And they do this for a surface code. So this is a superconducting circuit, they consider it at D3, so distance 3 patch.

Room: So you have essentially 9 data qubits, you have 8 ancilla qubits here. So you have to run these circuits that I’ve described. So you initially run the circuits that gives you the syndrome measurement. The syndrome is passed into what they call central feedback module. So that is the thing that contains the FPGA that implements the decoder, the neural network decoder in hardware. They do the decoding, then they, in some experiments, one version of the experiment, they pass that onto these pulse generators. The pulse generators generate the pulses, the pulses are then sent back to the chip. So you can plot the plotting essentially a fidelity of some initial state. versus the rounds of this error correction, the number of cycles that they’ve implemented.

Room: So this is still just a memory experiment, but in one variant, so you can look at the… the red curve, they actually do apply this active correction, okay, directly. So they’re not applying any gates, any logical gates, but they’re applying the correction, they’re fixing the errors. And you can see, so if they’re doing nothing, then it’s the blue curve. So the fidelity is lower than if they do something. And again, the green is the, if it’s when they Sort of shift that correction and the decoding until after the, you know, the full the full experiment. So these are some numbers, and sorry, this is probably a little bit small, but the classical only cycle time, which involves all those things that I was mentioning, that’s why I was mentioning earlier, it’s not just decoding, it’s a whole bunch of other stuff that has to happen.

Room: And then the quantum part, which just means running the extraction, the syndrome extraction circuit, that takes about 1.2 microseconds. So the total, the full cycle for them is just under 2 microseconds, which is sort of try to compare to some of the other results, and that is really state-of-the-art. Yeah, so QEC sort of improves performance versus doing nothing here, and they do talk a little bit about some limited non-Clifford operations. but please leave the paper if you’re interested. So the QEC does not directly fall under any of these application workflows that we’ve discussed earlier, but you know, because sort of the applications will run on top of. anything that’s being done here. One needs to this interleave interaction between the quantum and classical hardware.

Room: And I’ll skip all of these, but maybe one for the sake of discussion, one point that I might sort of ask, you know, down the road, presumably, once the technology advances, we can imagine a world where the application developers don’t have to care at all about the quantum error correction that’s sort of happening underneath, right? You will just define your circuits or maybe even some higher level programming language in terms of, let’s say, some logical gates and you sort of pass it on and then the hardware, the error correction just happens.

Room: But I sort of wonder, because that might take a while before we get there, but I sort of sort of wonder whether there are cases to think about where you sort of define an application and then you define some sort of quantum error correcting code, but you sort of combine the interaction a little bit more closely, right? You have to think about the details of how the error correction is done, and it’s tailored towards the application.

## QEC Discussion, Co-Location, and Decoder Scaling

Room: It’s maybe a little out of scope, but I was just thinking, but when you look at, like the last paper you showed out of China, I’ve seen some work where people are trying to squeeze a little bit more out of this whole process. and using syndrome data, not just for QAC, but also for calibration. And I was just wondering if you’ve seen that too, if you thought about the added complexities, but the kind of the nice two birds, almost one stone, that sort of approach. Yes, I personally haven’t seen this, but that would very much fall under some version of this, right, where you sort of utilize something else besides error correction. Like you think about the error correction, but you also think about the overarching goal, which could be calibration or could be particular application or whatever, right?

Room: So the first one, by the way, very nice talk and things that you laid out this, the workflow itself so nicely for that case. So I understand you’re right that this does not necessarily fall under this classical HPCQC case, right? How can can you can you project how an end user workflow? could probably have to interface with this. Yeah, I mean, so in the long term, the direct interfacing, I think, will be minimal, right? Like you sort of, once one implements this underlying fault tolerance and quantum error correcting mechanisms, then the user or the application layer doesn’t sort of directly have to know anything about it. It just says, I need to run this circuit. This is the logical version of the circuit. Some hardware below has to figure out, some software hardware has to figure out how to run this.

Room: But isn’t it like, so I think this is just a paper from IBM or so, I think. So where they talk about their hierarchical error correction. And so if I have fully understood it, that the argument there is that At higher levels, you probably have application-specific. Correction routines that. improve the overall workflow. I just meant in the long term, presumably that will just not be the case. My guess is, because you won’t have to, but in a short to me, it’s some version of kind of connecting the two. But I don’t know this IBM’s work, so maybe somebody else can chime in. When you say that near term, it may be an issue, the error correction, application specific error correction components that we have to account for near term. And then he was discussing the workflow layer for the user.

Room: I’m kind of curious about the hardware level. So if you’ve got classical, I’m sorry. Excuse me, if you’ve got classical hardware that is dedicated to keeping the quantum system healthy and alive and dealing with the error correction, the decoding, all of those components, and then you’ve got classical over here for the application side, if there is any sort of dependency, Right, for the application-specific error correction, right, then that would suggest. that latency is going to become a big issue quickly, or is it? And does locality then become an issue because of that parameter? So, I mean, that the first question was why only in the… in the long term that, why in a long term you might not need this sort of direct integration.

Room: This is, the argument is sort of, to me at least, a little bit like this is for the same reason that we don’t have to write assembly anymore, right? So like, Like it’s everything is good enough and fast enough that we can, the application developer doesn’t have to worry about those details. They just think about the logic of the application. From what Sebastian and Laura were saying and asking, I think like in the near term, the user might specify the required accuracy, which would change the QEC code that might be used, and then latency might become a problem, like Laura said.

Room: So what I can imagine is something like from classical software, for example, we have configurable parameters for error correction, depending on the type of application, because we are well aware, like in current systems, for example, in rate systems, we have like rate 0, rate 1, 0 + 1, and whatnot. And depending on what you need, you would have 10, okay, surface code for this part of application. I can imagine this in the short term and might still be in the midterm a problem where people actually want to exchange stuff themselves to figure out something better.

Room: And sort of connecting this to Laura’s point earlier, you know, like having active error correction that necessarily puts this constraint that is sort of not directly seen in some of the application layers that we’ve talked right? where you really, there is a sort of a hard time requirement that at the very least the decoding has to happen, right? So if that’s the case, then that sort of connects to our previous discussion. What implication does that have on having the machine living close to the sort of classical infrastructure?

Room: So, when we when we say classical, can we can we kind of the the error correction classical and the application classical, because you’re what you’re talking about right now is the the the, I mean, I’m sort of referring to both to some extent, but yeah, but I was mainly thinking about the… the classical related to the quantum error correction. Right, presumably this stuff here, if the latencies are really have to be that small of order, you know, hundreds of nanoseconds, let’s say, or something, then presumably you actually really need the classical hardware, this classical hardware, the FPGAs or GPUs or whatever to really live very close by.

Room: But then on the other hand, if then, so if we say, okay, let’s suppose that’s true, and then we really abstract the way quantum error correction from the logical operations, then maybe the classical computer can live elsewhere, because now it doesn’t matter anymore, and that’s a long-term thing. But if in the short term, the error correction or the the application layer can live somewhere else. But now, if, like, if we’re really thinking about this here, where the application layer now knows something about the error correction, and then maybe it has to therefore inherit some of the latencies that are associated with the slow latencies associated with the need for running quantum error correction, that maybe that induces A constraint that the application. classical application hardware also have to be somehow close by.

Room: I mean, there’s been this, there’s this kind of this overwhelming point that, you know, that the HPC system that the applications part can be anywhere, you know, the quantum can be here, et cetera. And that, and they’re basing that on like performance data for the, you know, for the between the quantum and the HPC, you know, for that part. But thinking about this parameter and this layer, if this does become a dependency, this literally changes how we place systems. And I’m hoping that on our ecosystem, we’re going to have a portfolio. of setups, right? Some of them may be remote, and then depending on application, you know, there may be certain systems that you have to go into because of, you know, whatever issues that you have.

Room: Because this is kind of one of those points that we need to kind of at least have a idea a little bit, you know, not a, that we have to not have a hard no. You know, at this point, we have to say, you know, this is something that we actually really do have to pay attention to. So first, actually, I really like this idea of QEC application co-design because… One of the big problems we are interested in is simulating Hamiltonians, is, you know, trotterized simulation of Hamiltonians, for example. And most Hamiltonians we are interested in simulating, they have translation invariants. So what ends up happening is that you can build error correcting codes that use the translation variants of the Hamiltonian to, you know, in the code automorphisms and so on.

Room: So there’s some really nice work from Kuera recently where they used this to basically co-design QEC LDPC codes together with the Hamiltonian simulation problem they’re interested in. So that’s the first. I think this is how future. applications, fault-tolerant applications should be developed. I don’t think all QPU vendors are interested in exposing their error correction layer and error correction stack. And I understand that, oh, you know, maybe you’re running some sort of gross code underneath and they let you run some concatenated. some simple classical code on top. These are real problems, but I do think that the kind of original model that I saw from Chris this morning, where you have a small HPC tightly coupled to a computer, co-located tightly coupled, and then a larger HPC. on campus, slightly loosely coupled.

Room: This solves a lot of these problems because, okay, let’s say you want to use a different kind of decoder, you activate the GPUs in your nearby small HPC system. So one thing I would really like, I would really like the large-scale HPC. Folks to to weigh in on this on this this localized snowball HPC system next to a, I mean I know I’ve heard that model too, that we placed you know a moderate size application based you know HPC with with the quantum and the classical error correction and all of that, and then you pipe you know there, but but when we’re doing, but when we’re thinking about, and maybe it works, I don’t know, but I would really. We want some large-scale folks to weigh in on that, if that would work with some of their runs.

Room: Perfect, and maybe I’ll make one more quick these things, then I don’t think there is much QEC R&D you can do other than just simulating circuit-level noise on your computer. you know, doing things by pen and paper. So if you really want real experiments, I think you do have to enable the GPUs, CPUs, the FPGAs in a nearby HPC rack to interact with your QPU and actually do experiments rather than just hard code the error correction layer into that, which I understand the reasoning for, but I don’t think that’s the right model at the moment. The co-located small or medium-sized HPC system. would not be sufficient for the error correction. I don’t know if you can confirm that, Peter.

Room: I mean, I would imagine that you’d still need to have some FPGA or something where it’s super low latency, but then maybe you would have the sort of the closer interaction with some. co-located HPC system, let’s say, that would somehow, maybe, I mean, I don’t know what that would look like, Ravi. Probably, in fact, if you’ve read the IBM’s paper, maybe that’s the 1st place to look at. Peter, can you go to the latency, the networking latency of this thing? Do we have a sense of how long it takes to send the syndrome data? To the decoder, yeah, so the the… So they defined as a syndrome calculation. So that is, I think, that’s related somehow to the something that happens after running the sort of the last step of the circuit and then before things get passed to the to the actual decoder.

Room: And what Chris said, I mean, his co-located, tightly coupled HPC, if I remember correctly, he was talking about 1 to 10 microseconds. I think your comment, I think the one part of the IBM paper was showing that they can do the error correction on the ASICs in time, right? But again, this is specific to superconducting qubits only because they are so fast. So depending on what kind of platform you choose, you might have a different type of scenario. Because I think at Continuum we have done this showcase where we had a NVIDIA rack that is mimicking the Snowball system doing the decoding for our system because the latency matters a lot less for us. It’s not just superconducting qubits, it’s also spin qubits. So I don’t think we can just be like, oh, it’s one modalities problem only.

Room: I just want to jump in, Laura, for the question that you had about, you know, now we’re starting to talk about setting up small piles of HPC systems. I think that comes down to a discussion we were having this morning is what do we define as tightly coupled and what’s loosely coupled? If they are tightly coupled, You don’t need to have this small pile of HPC resources. And then we can start to say, okay, on this loosely coupled one, yes, you might need something very localized. very low latency, high bandwidth to be able to do this. So sorry, so coupling equals latency? Well, it’s a more, coupling is a really broad term. So coupling would be bandwidth latency is something we have to work on, I think.

Room: And that’s where I think we need to, I think part of our group here, we should. think about making a better definition for that. So think about here on campus, you’ve got Frontier sitting in one building, you’ve got the IQM machine sitting in another building. Maybe we can collaboratively put together a First cut at that. Yeah, and I mean, so this is this is a point that Amir and I, we the coupling, the integration, the use of we’ve got and the latency.

Room: I mean, there’s been a couple of papers that have like defined, and we kind of need to kind of coalesce all of those, because the other thing, and I didn’t mention this in my talk, but you know, when I was listening to Gina and Rick Stevens and all at the Genesis thing, they’re talking about in that whole HPCAI, they’re talking about tight integration, right? And what they mean by tight integration kind of on this, you know, this federal, federated model, I mean, they’re talking about like policy alignment, administrative alignment. And I mean, I mean, you know, we can have a supercomputer in building A and a quantum computer in building B. And if the… the networking is not running or something, then you’re going to have weird handshake, you know, section points.

Room: And so it’s administratively a lot easier when everything is on the same network, when it’s on the same administration, when it’s administered by the same people, when the updates to the software on both sides, or, you know, and all of that’s aligned. that really affects the end-to-end workflow, right? So to me, that is tightly integrated when it kind of hits on several different levels, right? And then the coupling versus, I don’t even know if we should, maybe we should just kill coupling, I don’t know, and then, you know, latency and integration, something like that. But I agree, I think that that’s something that we really have to work with. work out because I see a lot of people getting really ruffled feathers on the integration argument when we’re actually talking about different definitions of it.

Room: Do we take, I mean, do we take one target application and try to run it through several different models and see, You know, where it would break, you know, try to do that exercise, or you know, really, how can we do this? Because I mean, the whole thing with my talk, you know, the locality, the workflows, the administration, you know, all this stuff, so you know, there’s a model, can we can we just have an application, run it through model A, where this system’s here, this system’s here? These, you know, this, it’s run by this sys admin and this update schedule and this and that and the other, and this is the policy and this is the worker and this is the manager.

Room: I mean, I know it’s kind of banal to the, you know, the really exciting technical work, you know, with the quantum and the HPC and the scheduling and all of that, but that’s what’s going to break the workflows. So how do we attack it? be super excited to do it, but we gotta do it as a community. Here, you know, as a for this particular work stream, you can take the action on the work stream. I think we do is we summarize it as something where we need to come up with a short-term action to start moving forward.

Room: I’ll just point out that so the part of this group is really actually sort of in the mission of this group is defining that those terms and and some of the meetings that we started having that Johannes is leading again is leading again who couldn’t actually be here. And part of this is because their application they’re concentrating on is maybe different, or maybe the hardware that they have has different end requirements. So it’s sort of tricky to get everybody on board, but that’s the goal.

Room: So, you know, as I, as I mentioned with the blueprint and the idea of 2028, you know, to, if a system is there and at certain level of fault tolerant and whatever, what sort of application if they do that, again, it’s one of those that I really want to push on, let’s figure, let’s try to figure out or at least get a better idea. about application specific error correction dependencies and all of that, because, and try to do it sooner rather than later, because again, I think with the push that we have in the community, you know, does the locality, do certain things actually make a difference? And we should try, we should try to have some sort of an answer to that before, like, before. systems get seeded, larger systems and stuff.

Room: Maybe I’ll also make a follow-up of that, which is that I think we have a very, so we have now a few very well identified applications, right? Simulating for me, Hubbard for material science, quantum chemistry, cryptography, and a few others. But we also have very good emergent error correction codes like surface code by now is very well understood, but also families of LDPC codes are being very well understood. But one of the things I haven’t seen enough of is understanding the decoding requirements of LDPC codes and in particular the hardware requirements, right? So As I think Peter was originally saying, you know, depending on what you do this decoding on, you know, you have different hardware hungry solutions. And I think getting a better sense of what those look like together with the application.

Room: So going back to the co-design, like pick your three favorite applications, pick your three favorite error correcting codes. do analysis of how they pair up and what the requirements are. Could you go into a little bit more detail about, you said we don’t know, is it the size requirements? So what happens is that usually when we say, oh, this classical hardware is responsible for the decoding of the error correction, that’s the one that, you know. is right next door to it and kind of plugged into the QPU. What they’re running is some class of algorithms which look something like minimum weight, perfect matching, or they’re a class of algorithms that we run. And we have a pretty good understanding of how hard they are to solve for.

Room: Specific. error correction codes, but as you change the error correction codes, the time to solution for those things, how much time the decoder will take to solve the decoding problem, tell you what correction you should apply, varies quite a bit. And so getting a better sort of, this is like a performance profiling of your decoder. And that’s what we need to understand better for LDPC codes. And to add to his point, basically we have machine learning solutions most of the times for these decoders, right? So we would have to look at, other than those requirements, we would have to look at different types of qubits. We would have to look at different types of parameters these models can handle. So thinking of the data plane, control plane, yeah, we have to understand the latencies there as well.

Room: So, I mean, there are different, like, for example, for FPGA, we have different decoders, but for GPUs, we have different decoders, and for ASICs, there are different requirements of the decoders, and some of these come with, you know, like in terms of FPGA, we would have to consider the GPU. blocks. And so hardware level configurations are different for each of these models, how we are fitting them in, how we are running the, training them. So everything changes if it is depending on the hardware, like the classical hardware.

Mohseni, Masoud: I don’t know if there is an audio for virtual attendance.

Mohseni, Masoud: When you consider real-time decoding, there are certain syndrome events that are rare but have to be handled. As you scale up the system, as you have

Mohseni, Masoud: basically larger number of logical qubits and you going to implementation of algorithms that needs very deep circuits. These rare events become, they are happening all the time, so you need a heterogeneous approach for decoding of different, you know, pre-decoder, like more standard decoding and… more fancy hardware for rare events. And that means that the entire architecture should account for that. It’s just a different workflow for when you scale the system to really, let’s say, 10s of thousands of logical qubits or bigger systems. And those are Definitely not be that be on GPU and most likely FPGAs.

Room: So as I was saying that you have this classical hardware that is running these decoding algorithms. And so there is a lot of variability in how quickly the decoder spits an answer. So imagine it’s like, okay, there was an error on a few qubits. You send it to the decoder, the decoder quickly solves the problem, tells you how to fix it. But now imagine there were errors on many, many qubits. It could take a very long time for the decoder to actually find the solution. And until then, you know, your qubits are waiting there. Of course, you’re doing other things with them to keep them alive, but then you have to wait till this decoder is done and then apply correction.

Room: So one of the ideas people have had is that, how about I build a pre-decoder which catches all the easy things, is very fast in the connection to your QPU. So for all the easy errors that happened, I quickly solve it, apply a correction. And then when the pre-decoder gets an error, which is like, oh, this is a very, it’s going to take a while to solve this problem, it sends it to a much more heavy hardware, specialized hardware, where it does all the decoding and then sends it back. So basically, you classify your error events as easy to solve, hard to solve, the easy to solve run on some lightweight hardware, and the hard to solve run on more heavyweight classical hardware. So as we agree, right, that the tight integration of classical resources is absolutely essential here.

Room: The point is that pretty much every, I consider this part of QEC research, like QEC hardware design architecture research. And in fact, like pretty much every QPU vendor is going to do this. Let’s say I allocate a bunch of FPGAs to do decoding. So what you really want to do is, from the hardware design part, from the system architecture point of view as well, you want to just allocate a couple of them as these pre-decoders that are always fixing your small problems. and only kick in the higher ones when the. But I’m just, I mean, to that point about the easy hard decoding part.

Room: So in this case, I’m using a very simple definition in the sense that We have a pretty good sense of what are easy syndromes versus hard syndromes, which is like kind of the error length, the chain length of the errors, which is how many errors, roughly speaking, happened. Pretty good sense of how these classifiers work; they’re a bit more classical problems actually, the the the what the decoder is working on, so we have a good classifiers for these things and and we can allocate lightweight hardware and heavyweight hardware.

Mohseni, Masoud: There is, in fact, you have to, you know, categorize it into 3 level of hardness of easy, intermediate, and really hard. Generally, this can be mapped to solving NP-hard problems in worst case scenario.

Mohseni, Masoud: And there’s just a, like you could say, parallel distribution of errors in a sense that a lot of them are very easy, but the median kind of syndrome events could be intermediate harm, and which you need, let’s say, GPUs, but then you might need to involve like even probabilistic computing and even ASIC solution for certain really hard instances of syndrome events. So I’m saying that it’s a three-tier hierarchy, I would say, when you scale up the system.

Room: The one thing that I’m also thinking about when we’re when we’re talking about the latencies and the integration based on latency, right? I mean, I’ve seen some numbers and, you know, the idea of like, how much compute would be available in late, but what I’m trying to understand is, not to be like so tactical, but what I’m really curious, can we get some sort of idea of what sort of latency would be actually involved for a particular error correction at, you know, so many logical qubits, whatnot, the, you know, the hardware required, but the distance required. I mean, literally, you know, based on that time, I would be very curious because both the size of the compute and the distance is making me wonder, first of all, what does that actually look like?

Room: And then if it’s… close, but a whole lot of compute, and the compute all has to be close. Are we now looking at like crazy stuff of like quantum computer with the HPC around it, you know, in disk or even three-dimensional? I mean, you know, we’re putting, we’re starting to stack up data centers. I think there is this paper from NERSC people where they estimate, like, let’s say if you have 100 logical qubits, the syndrome throughput is something like… Depending on how you, what kind of decoder you’re using, it’s like 1 to 50 petaflop per second, I think. I want to say that seems like a large number, maybe, but I think it’s 1 to 50 petaflop per second.

Room: But, but this, yeah, I should double check, but I’m pretty sure it sits in that range, so I can pull up the NERSC paper, and this is also on a surface code, so this is not on the more complicated error correction codes, and so, so, but, but wait a minute, sorry. And we, I know we have, we have these numbers internally to Alice and Bob. For the work for the OpenQSE working group, can you just share whether, like, the order of magnitude that he, well, like Laura, Laura was asking, I think that answer depends on how many logical qubits your system is, sure, the patch or whatever, yeah, so it’s not that high when you’re talking about… the first scientifically relevant 200 or so logical qubits. It’s not that scary, at least for our system.

Room: But it’s because yours is noise biased and it’s primarily bit flip errors. But you have to, but I’m just saying like your number depends on, for all the systems, it will depend on how many logical qubits you’re talking about. Maybe we take the action to maybe dedicate one of the meetings to continuing this. So the one thing is the reason that I think that this is important, and again, kind of going back to, I mean, in my mind, again, thinking about facilities, right? We’re not, the first system coming in might be quote lightweight, but it takes years and years to build centers and facilities for infrastructure, right? So the kind of idea about, yes, we’re going to get to scientifically relevant quantum computing in the mid 30s, how many logical qubits is that?

Room: I mean, that’s why that paper is really good, that blueprint. but then what is the HPC or the classical compute needed? What is the, you know, can we get an idea of the latencies, any sort of, even if it’s postulates, but we need to get, you know, enough of an approximation that we can as the facilities. I mean, I know that this is happening like in QBI and some other things, but that’s also, some of that information is also not coming to people who are actively working. on facility design. And I can think of, you know, a regional one that’s actually dealing with that issue right now, right? So this is kind of something that we, you know, it would be really interesting to have these discussions.

## Non-Performance Requirements, Traceability, and User Personas

Room: Yeah, so I mean, what I was, when I was talking to Amir, we were talking about kind of zeroing in on user groups and kind of these non-technical requirements. And for me, that would mean things like service and support. governance policy, anything that could potentially break a workflow. And I know that this is not a conversation, especially on the governance policy, reproducibility, tracking of the workflow through systems, particularly when we’re across different institutes and different environments. You know, we’re thinking about this in other spaces, but how this is going to work on the quantum side. And also, I mean, my biggest, one of my biggest concerns or thinking points is literally about reproducibility with hybrid systems, with a disaggregated quantum system. you know, across multiple institutions. Reproducibility and traceability. How are, where are we in the quantum space on this?

Room: Where, what can we be doing or what should we be thinking about in this realm? When you do quantum simulation for a really hard problem, which is what you probably want to do it for in the 1st place, most of them are not really verifiable, right? And if only some people have specialized hardware, it’s not reproducible. So is it even, I mean, I guess maybe the reproducibility is a bridge too far, but I think at least the traceability and the documentation, that’s going to be the first hurdle that we have to figure out in an ecosystem. I think settling on a set of problems that cover the space of the kinds of applications we’re thinking about would be great for the community to do, because if there’s a proliferation, it’s really hard to nail down reproducibility.

Room: Yeah, we really, it really seems just from the other conversation too, portfolio of candidate applications. So I think yours, you know, would be a good one to look at based on the blueprint, something, you know, really kind of fundamentally in this, in the quantum chemistry environment, that’s kind of first up, you know, what we can deal with. But you’re right, we do, and we need to have a a portfolio of applications that have coarsely different enough behaviors so that we get a bit of a spectrum. And I think settling on what are we comparing to is an important thing. Like what are the standard results? in, you know, quantum chemistry, we can generate lots of results on a classical machine, computational fluid dynamics, optimization problems. It’d be great to have a set that, you know, we could go to kind of thing.

Mohseni, Masoud: One would be that you test it on a smaller system and you do just brute force simulation that says a state vector simulation up to a certain size. And then you assume if the physics do not change, then it’s, you know, the same outcome is expected if there is not a new physics at the larger size. The other one is that you dilute the number of T gates and test, do efficient simulation with the smaller number of T gates, and then you expect that the outcome is the same for larger number. There’s another one with the entanglement truncation like tensor network techniques. But I think the 4th category, the most interesting, which is you predict a new material or a new system and you just do experiments in the lab. And if the prediction comes true, it means that the simulations work.

Mohseni, Masoud: It’s a direct verification in real world. I think there is a fifth category with AI, but I think that’s just entirely different working session to discuss.

Room: I would say IBM just did that with a new molecule designed on the quantum computer, and they synthesized it. Somebody synthesized it, so verification might look that one up.

Mohseni, Masoud: It would be a verification which is basically interpolating in latent space of a foundation model. I think that could be it, but this would be indirect.

Room: And I suspect you’re probably one year too early, but I think it’s great that putting this on the table is something we need to start thinking and really considering what to do. I think we’re also seeing Oak Ridge is doing a lot of work with trying to verify even like the 1D models, make sure that the model matches the actual chemistry. So I think once a lot of that work is done and we have better results, I think we can really start to apply that to what we might want to do for governance policy verification, all of that.

Room: So for the IM policies, for the IM policies like identity and access control kind of things, we can definitely have like, you know, tokenization, not the AI one, but like JWT tokens and, you know, authentication between the different user personas, which you were kind of explaining about in the beginning. We can have like one model which would basically store, you know, all the user personas and then use that for authentication, like how we do it in classical world to answer that identity and across access control thing. And then similarly for data classification, we can use those user personas and you don’t give access to specific like accessibility.

Room: We can give access to specific user groups, like researchers would have access to some specific architecture of. or like open QSE’s architecture and then who are our end users would have different roles and they would be able to access different things in our architecture. For sustainability reporting. Like we were looking at different QPU vendors and what their budget, not the budget, but like power usage and electricity usage look like. We have iDRAC, which is our remote access controller. So we maintain the energy telemetry for, like whenever you are going to access any servers or you are creating an HPC environment or a cluster, you would access that through this iDRAC. And then you can actually store that telemetry. and it will tell you on the basis of what your workload is, what that telemetry looks like.

Room: And so similarly, we can have that for quantum computers, whatever workload we are sending. They have sustainability reporting and all we have to do is, you know, in a dashboard kind of. relay that information on to the HPC side or on or yeah on the user side. For sustainability, okay. Yeah, I came from the telemetry for operations for scheduling a little bit the same the same idea, yeah. I know governance and policy is a bit boring, sorry. In terms of the users, since I got some interest in that one, let me go back to this one for a moment. Like I said, it was kind of a first hit with… some interviews, and I really want to expand this out.

Room: So I guess the first question that I really have is among us, are there other teams or other folks doing any sort of structured user persona work? Business development team and marketing teams are absolutely have done this, but I kind of I kind of have 1 foot on that side of our org and 1 foot in the physics and engineering side, and when so when I look on the the where the rubber meets the road and I look at the actual teams who are working together. on our government programs to do simulation work. I would recommend, we haven’t done this yet in Alice and Bob, but I would recommend a different approach to this, which is to look at the teams who are intimately working together. For example, like ARPA-E has the QC cubed. program for computational chemistry.

Room: And I would hypothesize that all 10 of these teams probably look the same. From our view, we have, at Alison Bob, we have fault-tolerant algorithm designers and one partner organization. at LANL, actually. We have folks who are doing, who are experts in tensor network methods for getting the initial state for our problem and optimizing the circuit to instantiate that state. And then we have from University of Michigan. an expert in classical computing who’s able to try to separate the weakly correlated from the strongly correlated parts of our quantum chemistry problem. And these three users can’t really operate without each other. And they form this, you know, deeply integrated team of, in a sense, a user team of a QHPC system. And I think if you tried to cut them into separate users, they wouldn’t be users.

Room: That’s a really, really good point because, yeah, when we’re also thinking about, yeah, you’re absolutely right when you’re thinking about like even some of the… design choices for a quantum HPC solution that you’re procuring, right? In that you’re procuring, you would be basing it on the input of several different teams. Hopefully I didn’t actually misunderstand your question, but Oakridge, if you’re trying to understand like what the users are doing, is this some version of what you’re asking or? Yeah, what they’re doing, but what their actual needs are, what sort of, I mean, so it’s kind of a whole profile. And she’s, you know, she is a, she’s actually working on HPCQC workflow or problems of some sort.

Room: So, But then, but that’s going to be different from, you know, Matthias, who is a ******** physicist who is, you know, wrangling with the qubits, you know, on or using it as an experiment, you know what I mean? And so, and so those tools, that whole tool, and it’s also going to go to the workflow conversation to the workflow team, right? So, when we define these workloads, who are actually these, who’s the personas of these folks doing it? And then, and then also, you know, where is their experience level, where, what sort of tools, what sort of environments do they need?

Room: This all, because the other thing is too, is that there is a difference in a quantum chemistry application between, you know, Matthias, the ******** physicist doing it and, you know, Jenny doing it here at the integration thing and Bob doing it over here with, you know, a clicking a workflow interface. It’s like the applications themselves also have different layers. Yeah, so at least some number of the things that you mentioned, there’s like an active effort here of people who are involved in this QCUP program where every year we send out a survey. to all the users of QCup and then the server is fairly detailed. So it sort of tries to understand the level of knowledge, the application frameworks people use, the types of applications they want to run. And I think there’s an active effort even to sort of learn more information now.

Room: So I mean, I can kind of just wrap this up because I think we have to go to the next one. You know, I would really, if if you’re not doing it, but somebody on your team at your company is doing it. If this, this is market research, so what the companies would call in some ways, for others, for the user facilities, it’s, you know, to better understand how we service the users, but I’m trying to bring that information all together in a way that’s not just a pure market analysis, but that more has technical depth for, you know, how we support the workflows and then how that would actually potentially even cause any sort of design considerations that we would have to think about.

## Cross-Workstream Coordination and System Architecture Actions

Room: Well, I think what Amir would like us to do is, I mean, later on we’re going to have, we have to create a report out and basically report back to the OpenQSE panel. And Amir is interested in As he said early on in three things, he asks the individual work streams to generate milestones and the prioritized list of, you know, next actions or potential deliverables. And I think the challenge now would be, you know, whether we agree on, whether we identify such milestones, agree on such milestones, and, you know, can note those down for the report out. And the second thing he asks us for is, to identify potential overlaps with the other work streams. And the other work streams are software architecture. Quantum resource in quantum resource interface and compiler infrastructure.

Room: I mean, well, we have not really discussed, but when we talked about quantum error correction system requirements, and I don’t know, there, I guess, you know, there might be some opinions in the room on… on potential overlap to fault tolerant compilation. Quantum Resource Interface Group deals also with integration with HPC schedulers. So I can imagine that workflows we discussed here also have an overlap with… HPC schedulers. So maybe a first step to better integrate or overlap with the other work streams is to find areas where we are not speaking the same language and then smooth that out. It’s always just hard to figure out, like opening up a blank sheet and like trying to do the vocabulary. And also we’ve already seen like within our own group too, you know, there’s also vocabulary ones.

Room: And the I guess the thing that, I mean, with workflows kind of represents a global, right, because it brings together a lot of those pieces. And so, and then like what Tom was mentioning is, I mean, basically kind of having a mini co-design. community within our own ranks where we have some portfolio applications that we just need something, we need to be able to tell a story, right? And then, and kind of, and then basically, you know, looking at like the blueprint document and like this is where we’re supposed to be in this period of time.

Room: These are the sort of applications that they’re suggesting, if we have other ideas, of course, but you know, kind of take those and just start running with it, you know, from the workflow perspective, how does it hit the, you know, how does it affect the actual, you know, architecture? How does it hit the actual system software? Because when work when certain things happen in a workflow, if we don’t have the compiler like this or the compiler like that, it causes a… problem. It kind of feels like we maybe need to have a couple of globals that kind of tie us all together. So I don’t know if that’s like a missing working group or I don’t even know it’s a working group, but it’s a kind of a crosscut.

Room: So if I understand you correctly, then this would mean that this working group could… prioritize or pre-select a couple of exemplary workflows. That would then end, and yeah, and come up probably already with some high level, at least, system requirements. These workflows would be. Well, demand, then, in the next step, we should, with this output, inform. This software architecture working group. Schedulers and yeah, with compilers, I’m not so sure. When we talk about personas, the comment that I made before, so we’re kind of tying a few things together. So like statement one, you know, the blueprint is saying that maybe the first things that we can attack with a certain number of Qubits, you know, a logical or logical qubits is quantum chemistry of a certain sort, whatever.

Room: Okay, then statement two, at some point, you know, I was mentioning that there are, you know, for an application go through, there’s going to be different interpretations, different needs based on user.

Room: So statement three, The idea that we’re going that we want to hit kind of a threshold mark, you know, early on, so the power users are going to are going to be the first ones having results on an early fault tolerant system with their application, but we want to be able to make sure that once all everybody’s all excited, then you know, then the… kind of, I was like the early adopters, but then kind of like the next phase of people, the next phase of users, they’re going to have different requirements for their workflow of the same sort of application that can function on that level of qubits or that level of logical qubits. So what is their workflow? What are their needs? And then are we preparing that environment?

Room: So it’s kind of, yeah, it’s kind of the immediate need for the power users, what their needs are for that particular workflow that can be, or that particular application, like quantum chemistry, whatever, that can be supported by those level of qubits, logical qubits. And then the next wave of users are going to have a workflow that’s going to be different because they’re going to need certain things. And so we’re kind of systematically knocking back what those are. That would seem, that would seem something very progressive or at least, you know, getting the check boxes done. So, yeah, exactly, kind of… user persona based workflows for a particular application set like quantum chemistry that can be achievable with the first wave of early fault tolerant quantum systems that we’re going to have.

Room: Maybe to add to that, I think it’d be nice to, you know, do one of these end-to-end analyses where, you know, you’re like, okay, this is exactly, like this is a good estimate how much classical compute I need, what are the networking latencies, the decoding, how much decoding hardware I need for this. Two 100 logical qubit chemistry experiment that I can do, and then you know, put everything together in like a nice document, and then… Yeah, and then just, yeah, and then so just also thinking about that, like the workflow and the behavior needs of like a quantum chemistry. I mean, I’m just kind of curious if we can kind of take one example that kind of, it’s the same problem, but gets us, you know, kind of different levels of results based on the workflow that we apply to it, right?

Room: And then how it goes, because then some users are not going to care. You’ve got users that are like the power users, they’re going to look for certain results certain ways, but then somebody in like the biopharmaceutical area, you know, they’re not going to need that level of detail. They’re going to need something else. and they’re going to need some sort of interfaces and whatnot. And so that result is actually going to be a little bit different, but it’s kind of solving the same general area, right? And I kind of feel like going after something specific in quantum chemistry would be really interesting.

Room: And this would really support what the blueprint is trying to do too, because then you’re going to have, you know, these end-to-end workflow designs at a couple of different levels that accounts for the power user, but then the next wave, but then the next wave, right? And then with that, you kind of already get an infrastructure set up that you can go, once you start getting into the Hubbard models and, you know, eventually Fomoco and all of that, you’ve got, you know, you have your hierarchy starting to be built up in your infrastructure. What I also want to see, and that also came from the exascale computing report, I remember that they were measuring integration. You know, they brought that into actually their metrics. So, you know, we talk about benchmarking quantum systems and doing this and whatnot.

Room: We have all these metrics that are disparate. We also need to kind of figure out a metric suite for that actual end-to-end workflows and what that means. And then if that actually changes based on, you know, parameters of co-locality or whatever else. I think we need better metrics than qubit numbers and fidelities for sure. So for the metrics specifically, we have a lot of integration with resources group. They are already thinking about, you know, having a layer like QDMI or not QRMI for that purpose. No, just the QDMI, specifically because they look at some of the telemetry, and we can use that telemetry for guiding our, you know, metrics that we are that we are talking about, and it also has like a dashboard in QDMI which… actually looks at classical telemetry and quantum telemetry.

Room: And that goes beyond qubits and two qubit gates, fidelity, you know, it also looks at, I think, calibration data. So we can take, you know, make use of D1, D2 times and all sorts of things. And also about, sorry, just one more thing, about the user directed workflows. Don’t you think like if two users have the same problem, we will be wasting resources? If like 2 personas are targeting the same problem? and there are certain components which kind of overlap. For example, in SQD, you have a circuit like preparation in terms of Hamiltonian encoding or like probability distribution into a circuit, for example. Those are some common components which would be repetitive if we have user-based. you know, workflows.

Room: Well, I mean, I guess what I meant by that is, imagine, so imagine we have the power user, right, who’s going to go directly to quantum hardware, deal with it at a very raw level. They may have particular workflows. I don’t stand by it at all, but then you’ve got another group of users that, you know, need a higher abstraction level, you know, for whatever layer they’re doing. So basically, what is that, right? And so you’re still, because of the work from the power user, you’re still getting results and you’re still getting, you know, input and guidance into how, or you’re still building stuff. that you may build on top of, or it could be different. I’m speaking very theoretically right now, but I just, I think it’s important that we look at the different user personas and then potentially across the same application workflow.

Room: Maybe I can maybe I can formalize that that thought a little bit in the context of chemistry as you had in mind, right? So imagine that I have some chemistry Hamilton in some fermionic system, right? You can be like, oh, maybe an advanced user wants to use a very specific way they want to encode fermions onto qubits. But maybe someone who’s new to the thing is just like, okay, the compiler comes with a default encoding. Similarly, maybe a pro user wants to choose their own error connecting code. You know, they’re like, you know what, I think the default error correcting code on the machine. is maybe 2 bare bones, but I know I have my own error correction code, which I know will perform better.

Room: You know, as you’re just saying that, and now I feel a little bit dumb, as you’re just saying that, you’re kind of hitting the pattern that we had for the HPC users, actually. I mean, now that I’m thinking about it, because like some wanted kind of clickable interfaces, some wanted, you know, the best that they would do is, you know, use a library, you know, or they wouldn’t even get to terminal, you know, to command line. Some would like, you know, alter directives, some would, you know, go into like full pragma. I mean, so the thing is, you know, you kind of have like different layers and that kind of represents some of this functional abstraction, right?

Room: And maybe can we make this like a thing, like one of the to-dos from this thing, where at the end we just try to write this nicely in terms of different personas and different black boxes they can open and something, different abstractions they want to look up. Let’s bring the first one is the schedule of today for our work stream so that we can remind everybody about what we did. One example is that it was very clear that we were lacking, even within our own work stream, a common language, for example. And we can take those into the weekly meetings to then come up with more specific plans. Yeah, I mean the vocabulary one, and I think, and well, not so much this is the vocabulary, but concretely operational definition of tight integration. Actually, just integration in general, what do we actually mean by that?

Room: And then what would we consider tightly integrated versus loosely integrated and let’s keep the latency like as a component of that, but it means it means more. Yeah, you know, similar, you know, similar networks, similar administrative regimes, policy regimes, basically all of that kind of really super boring stuff that goes, you know, to the attorneys and to the You know, business developers or whatever to to hammer out to sort out, and it’s you know somebody somebody not updating a software package on one side and it not being and it being updated on the other side and it breaks the workflow, that that sort of stuff. From the open QSE point of view, I mean, this is more of a question and then I will make a comment.

Room: I assume we want some sort of unified framework that supports many modalities in a roughly homogeneous way, as much as is possible. To answer questions of what integration and latencies mean, I think it’ll be good to have a modality dependent time scale, right? So for example, when I talk about coherence time of superconducting qubits, we’re all thinking microseconds or 10s of microseconds. But when I talk about coherence times of trapped ions and neutral atoms, you’re thinking. Maybe a few hundreds of milliseconds or something of that 10s of classes, yeah, sorry, go ahead, classes of coherent, yeah, and that’s a really good ’cause I mean, you also don’t wanna be like constantly saying, you know, this ion trap or this, you know, whatever, some sort of boundary of classes of exactly, so I’m thinking there is the physical. qubit coherence time.

Room: Once you specify a modality, have a range in mind. I think these are the discussions we should have in our meetings, which is great. Are there any other key observations anyone would like to add to the list that we’re going to present to the rest of the community? how to enable QEC research where you know it’s not hard-coded on the FPGA. Like if you want to try your own QEC code, can you? But can a pro user do the same? Can you sharpen that a bit for me so that I’m not entirely sure I grasp this in practice? First, you need your fault tolerant compiler to be able to specify exactly how to compile it down. from the logical layer to the physical layer. And if the QPU vendor gives you a FTQC compiler, well, then you don’t have a choice.

Room: You’re stuck with error correcting code. The second is the choice of your decoder, right? It’s also a function, what’s a good decoder for a type of error correcting code.

Groszkowski, Peter: The second is the choice of your decoder. And so, if, if it’s arguing on SPGA, you don’t get to play around with the decoder.

Room: If it’s hard coded on an FPGA and you don’t get to play around with the decoder, then then you’re stuck with the whatever static correcting code they gave you. So if I rephrase that, is that that we should come to some sort of an opinion or do a survey on what the architecture should look like to support this type of research, given the fact that we’re not entirely sure?

Groszkowski, Peter: So, so it’s not very rephrase that is that we should come to some sort of an opinion or do a survey on what the architecture should look like to support this type of research, given the fact that we’re not adhere to sure.

Room: Because in my eyes, OpenQSE is here to help with standards and recommendations around enabling.

Room: How do you build a system where you can do QEC research? Right, okay, so we need to better understand the architectures that will support that type of research. And then we can make it specific, like, can you specify the error correcting code in your FTQC compiler? Can you specify the decoding algorithm? Can you specify the hardware that the decoder should be using?

Groszkowski, Peter: But we, we need to better understand the architecture that will support that type of, and then we may specify. Can you specify the decoding algorithm? Can you specify the hardware that the decoder should be using?

Room: And, of course, if you had like a very nice heterogeneous architecture, you could do it, you know, with the right networking, but if not, then you know, it’s just a thing that’s not available to you.

Groszkowski, Peter: And, of course, if you have a very nice architecture.

Room: I remember there was an action item earlier, but I’ll be honest, I missed my last cup of coffee and I forgot, right?

Room: Someone in the back pointed it out, but I don’t think they’re sitting there anymore.

Room: Also, it’s an extension of the common language, and so we have to define the what’s this workflow. I mean, you know, even the definition of workflow is pretty vague as well, so maybe we can divide subdivide it by application and workflow. Even, you know, we don’t know yet the we’re gonna include the, you know, error corrections workflow. All right, so is it really workflow or I think maybe it’s underneath the, you know, general workflow that the research scientists really needed so that we can define it something different or something like that. Because I think it was your comment before that you said we were the end-to-end work stream, right? And we have to understand the integration point, so yeah. I think you were suggesting that we should do it per user, right? Per user persona.

Room: I mean, not getting crazy with a ton of them, but a couple, a couple of representative user-influenced or user-driven. Yeah. workflow for the same. sort of application to, is that what we’re talking, to kind of see the diverge, that was the whole power user first and then second wave and then third wave because it, because the, again, for the quantum chemistry, for example, we’re thinking that the first wave of early fault tolerant, we could start getting some interesting results. Right, and for that, my concern was that there might be some overlaps between the actual functionalities that we are implementing per user. So there might be repetition of, you know, wastage of resources, possibly. I think it’s a fair concern. I don’t know, premium access to a device or some sort of set of resources.

Room: And then from there, if we want to share that application with others to repeat it, each one requires a different level of maturity, but it’s the same application, it’s just a different workflow. So me coding something up, testing a device, checking it out, then me saying, okay, well, I believe that my algorithm makes sense. I think other people are going to use it in a module that they then apply for more work means that that has to be. Um, a third point came from Peter, um, the… Application specific QEC relation. Yeah, so I mean that, yeah, so that one I would have put a pin in because I think that that’s that whole, that that doesn’t just, I mean, that affects potentially architecture design and location and all that other stuff.

Room: So I mean, if we need examples, then I think it’s like integration or tight integration, tight versus looser. Workflows, workflows, latency, yes, and application, and like redefine our whole field. And if I throw one more observation into the ring, I had the impression in the… You know, people that work on algorithms are obviously super enthusiastic about their algorithms and the algorithmic details. And it’s so it’s still a challenge to abstract, to distill the workflow patterns out of these algorithms. Can somebody confirm or disagree or? I’ll confirm that. I think we got excited and ran a little bit down the, you know, what are the specifics of your algorithm and implementation rather than making sure that we focused on the workflow portion of it. That may not be a workflow question.

Room: So I have an algorithm that I implement, and it goes between the different types of resources in different ways. Or let’s say, I mean, circuit features, or let’s say, required features of a QPU versus what is the computational cost to run that, right? Because when you, in my, at least in my simple mind, when you kind of want to come up with a decent system architecture for a certain workflow, then you have to understand what are the actual components of the workflow. I mean, how much classical compute, how much quantum compute, how much, what’s the combination? Is bandwidth that we’re, do we need to configure that at all? Don’t forget your application specific or application, whatever influence. I, I think there is a how to distill workflows from applications point, and ohh, whose screen is that? And I think you phrased it fairly well, right?

Room: Is we’re struggling to… distinguish between workflows and applications. Because, as the as the as the people working on the application, you get very excited, but then you may not realize that, ohh, these two applications you have from the workflow point of view, they’re the same, I mean. Like if we look at the dictionary definition of workflows and applications, applications is what SQD and credit risk analysis is and workflows is something that goes inside, like how we transfer or communicate between classical and quantum components. So I guess what we are talking about is essentially applications. Not really workflows. Can workflows are inside that? Maybe we have to define workflows, applications, and architectures. Because I mean, the thing is, you see orchestration, you see workflow. And so I think that, I mean, it sounds crazy, but the thing is, this is not unique to us.

Room: You know, this is pick an ecosystem, you know, so. Maybe I had another observation, which I think there was a lot of discussion around QEC requirements, including classical computation, coupling, hardware. I think this is probably what we discussed the most, and so, yes, understanding that, how should we formulate that? that bullet point. Hardware, software integration requirements of QVC. This is a, yeah, the third key observation. I think this is the third key. Hardware, software, and integration requirements for QEC. Resource estimates at the system level. User persona dependent chemistry workflow, right? So for the goals and milestones, we should definitely connect with resource estimation working group and figure out the QEC part of it for collecting the telemetry, the system requirements, like hardware and software system requirements. So they will be the best people to kind of, you know, what’s the bullet point?

Room: So connect with resource estimation group to outline the hardware and system requirements into a layer maybe. Wait, when I think about that for a second, and you had it formulated, so what was yours again? User persona dependent quantum chemistry workflows. Chemistry workflows. Quantum chemistry. Thank you, quantum chemistry workflows, and in the bracket you can write, we need, we need like scope it or define it, right? No, it’s just that it’s one of the first expected anticipated applications on a scientifically relevant one, Peter. This seems to be the application that we could get a success story out of to show we’re on a progress pathway. Instead of doing, instead of, I mean, we could take other ones, we could take Hubbard, we could do something with Fomoco, but it, but the, you know, the results are going to be just maybe not as impressive or exciting.

Room: Yeah, maybe one more thing in that was that can you add the end-to-end estimation, because yeah, we also talked about like how much classical compute metric or end-to-end, what are we gonna call it? End-to-end resource estimate. Should it be end-to-end resource estimate of user personal dependent quantum chemistry workflow? And then I think to your point, we start there and if it turns out not to be a good one, I don’t. So as I understood it, kind of the, so we realized, okay, we cannot kind of create a complete for the entire application soon. We cannot come up with system, you know, requirements or a proposal for. for a system architecture. So what we instead do, we pick one. how you call this field of class of applications, field of applications, and that is, let’s say, quantum chemistry.

Room: And for the early fault tolerant quantum computing case, we were kind of, we performed this entire exercise of end-to-end user dependent, yeah, yeah, yeah. And the thing is, I mean, there’s already Good use cases out there, right? We, I mean, you know, Tom has one, I, I, there’s other ones, Peter Coveney has some stuff, we’ve, I mean, there’s an IBM did some great stuff, so I mean, there are these cases, I think we just need to pick, you know, one or two of them in that same class, and then it’s always good to pick, you know, folks that are super friendly or teams that are friendly and easy and accessible to work with, right? because then we can get under the hood of what they’re already doing and start, you know, doing that workflow and all of that.

Room: I think we also discuss about the scheduling stuff. Maybe you want to add one of the scheduling. So maybe that might be integrated with the resource manager, right? So how we can add this like a workflow schedulers integrated with the resource manager, right? Or does that fall under the larger bullet point of, I think you said it, operational terminology that we need to then define and then classify? If we’re studying the end-to-end workflow, right, and with the particular problem that’s being worked on, if the scheduling component becomes an issue or a factor, then it’s kind of subsumed inside that workflow analysis, but or am I understanding? I think this is related to also the resource estimation, right? Obviously, then we can, you know, maximize resource utilization.

Room: So, you know, how this resource estimation starting from resource estimation and then how we can schedule it, and also we really define, you know, this kind of workflow requires this much resources, and then we can schedule it this way. Things like that, so, so we are already addressing that in the other groups, because software architecture is kind of focusing on collating all of that, and then resource, yeah, that’s a good, exactly, that’s okay, that’s good, and that’s, yeah, exactly, so we should actually make maybe a sub-bullet. point and exactly make this call out, you did Smriti, so that these are then, I mean, these are identified overlaps with the other working groups. It’s about scheduling, it’s about, sorry, you said it, Smriti, you had a good architecture, software architecture working group, and then resources and compilers is another working group, so they can.

Room: Actually, resources and compilers could have resource estimation, and software architectures would have the scheduling. Isn’t it resources and interfaces? Compilers is… You know, runtime and interfaces is one? It’s resource interfaces. On resource interfaces, yeah, interfaces, they they work on the schedulers, then we have a compiler working group, and then we have the software architecture working group, where the runtime. discussions. So software architecture then has the scheduling and then resources and interface. Resource interface have scheduling. They discussed today the latest QRMI paper where they integrated with, I don’t know, a handful of resource managers. So from what I hear, I think there’s two action items and you know, push back if you. feel differently, but I think there is a set of operational terminology that we need to establish. And we can do that in the context of trying to structure this end-to-end.

Room: Workflow right in collaboration with the others, so. The thing is, we’re doing workflows. Like we do, I guess my thing is we kind of have a overarching in a way, and then we also, I’m kind of what I’m hearing, we also have kind of specific things for clarifying, you know, even the workflow writ large, right? Maybe, maybe our job is, I mean, I don’t want to like elevate ourselves on a on a diagram, feel free, but but but do you know what I mean? It’s like maybe, maybe, maybe it’s our job to kind of create kind of target problems right with this workflow and how is it going to hit the schedulers? How is it going to hit the resource managers? How is it going to hit? the compiler, all of these things.

Room: And so maybe that, maybe we kind of have these, you know, figure out a couple of things and say, hey guys, we need you to all use this terminology or can we do this, that sort of thing. And then we’ve got this workflow where we’re, you know, going there. Their feedback and their whatnot, yeah. So I mean, the thing is, I don’t want to say like introvert, extrovert, but maybe we’re the social ones that we’re constantly going back and forth and making sure that we’re getting that information for them and then reporting it out back to them of how it all works together with the workflow. Maybe instead of sharing requirements, gather requirements. Gather requirements and feedback, gather and share, gather and share requirements, wait tight, because if you finish this in 50 seconds, we’re on time.

## Compiler Working Group Report-Out

Room: So I’m going to do the report out for the compiler working group. So one of the things we did was talk about terminology, just so that we can all. communicate effectively. So we want to use the terms compiler IR and interchange format. A compiler IR is a circuit representation that’s used during compilation, primarily an in-memory representation. An interchange format is a circuit representation used to connect different systems that’s primarily serialized. And then we also agreed on using the term compile in preference to the term transpile as a general rule. We have identified fault tolerance. quantum computing compilation or just fault tolerant compilation has not been fully demonstrated. Each QP vendor will need a different circuit format as input, at least that’s how it is today. And then SDKs have different circuit formats as output.

Room: And then as far as our goals, our scope will include fault tolerant quantum computing and quantum error correction and compilation concerns around those. Our scope will include handling hybrid quantum classical circuits. And then we expect the algorithms or applications will need to customize compilation and that we can provide that with the coarse grain idea. So in reality, the compiler will consist of many passes that do important things and cannot be separated. But in a coarse grain view, there will be a few small number of stages, maybe 4 stages, and we can allow plugins that customize compilation when people need to do it. We’re going to have the goal of defining an open QSE preferred compiler and intermediate representation. We expect that there will be multiple intermediate representations needed, especially for fault-tolerant quantum computing.

Room: And we’re going to focus on making a decision as to which IRs we will call open QSE preferred when they’re relevant for interfaces with our compiler reference implementation or interface efforts generally. And we also have the goal of defining A preferred, an open QSE preferred circuit interchange format. In terms of interaction with other working groups for the resource management or circuit launching interface, we’ll need some kind of abstract representation of the hardware constraints in order to support compilation. and a bunch of other stuff that we’ll be working on that we didn’t have time to go through in our session today, but we have additional things. Generally speaking, we’d like to follow Qiskit’s lead on distinguishing between required, optional, and custom. things. So a required thing would be something that we must have in order to work with.

Room: So in our case, number of qubits, what kind of modality, and what interchange formats does the QPU accept. We have to know that in order to have the compiler component do its job. Optional information might be data about individual qubits, like how noisy they are today or what operations individual qubits support. And then custom information would be for the case where maybe a QPU vendor needs to extend. our compilation component to do something special for their QPU only. So custom would be a way to classify data that we might request from the QPU and then connect that data to a custom component that is designed to work with that. For milestones, for the six months away, we’re aiming to make a survey paper.

Room: The survey paper is about quantum computing compilers, intermediate representations, passes, and requirements on circuits dictated by quantum error correction strategies that people are expecting. And then in the later half of the 12-month period, our milestones will be a draft specification of interfaces for our compiler component and a prototype for a reference implementation. And then in the deliverables, here’s some more detail about what would go into that survey paper. We’re trying to understand more about the existing quantum compilers, understand, describe what criteria we’ve used to select a preferred intermediate representation. Think about what intermediate representations support quantum classical capabilities and what would need to be extended. Same kind of thing for FTQC, although it’s not on the slide. Think about to what degree is there convergence on the functionality or intermediate representations and where is it very different.

Room: And then other deliverables, compiler related requirements for the overall architecture documents, Amir’s working on specification of our compiler component in terms of interfaces in the reference implementation I already mentioned. And then our working group sort of process and participants, we plan to meet every two weeks for about an hour. There’s me, representative from Quantinium, from representatives from Oak Ridge, and students and hopefully somebody from Alice and Bob, although we don’t know who, what their, which individual it will be. And we’re pretty sure we’ll have somebody from Amazon bracket, but we don’t know exactly what their name is yet.

Martin Schulz: Can you say something about, you said that you expect that you need different compiler passes for different algorithms and applications, so there’s a dependency which kind of makes the compiler part of the application.

Room: We, in our sort of collective experience, we’ve seen cases where people are doing research that’s sort of cutting edge enough that they need to customize the compilation in order to reach their research goals. They might be for example, making some kind of optimization that makes their circuits practical that they wouldn’t be able to run without. And so this is an important capability for sort of flexibility for researchers, but not necessarily something that everyone will need.

Martin Schulz: Do you see this potentially as a kind of a transitory thing until we have enough experience and then we can make general compilers?

Room: Yes, I mean, I think general compilers also, like for example, LLVM compilers definitely in the category of general and well understood compiler. It has this idiom of extension points within it so that it can be customized. So I think that can be a long-term thing, but the usage of it is probably going to always be for people who are more in the research category, as it is for those extension points for LLVM today.

## System Architecture Report-Out

Room: System, uh, systems architecture. The first is we wanted to review with everybody what it is that we did today, how we kind of drove our discussions, and then we have a couple of slides for you. That. point out some key observations and actions that our workstream would like to take. So we started the first session with just kind of an intro so that those who joined our workstream discussion understood what it is that we were trying to achieve here. And then we tried to seed the conversation with three provocative, what we thought were provocative statements, right? Three examples, so the intent was 3 lightning talks, each one a particular application and workflow that could get us started with with. Just exploring the space and understanding what might be important for us to work on.

Room: We did restrict ourselves to the assumption that we were working with logical qubits at this point, just to streamline things. And then after that, in the afternoon session, we had a lightning talk on quantum error correction. And they kind of opened up the aperture a little bit to start having a discussion about that level of integration. From there, Laura gave a quick presentation on non-performance related requirements, and we had a quick discussion about that, and then we kind of wrapped up for the day. I know that the community is still kind of evolving, and so we thought it would be important to share how we tried to organize ourselves. So we discovered that within our group, this is also a pervasive issue that we need to think about.

Room: This came down to as we were talking about workflows, we realized there’s some divergence in how we’re interpreting and our perspective of workflows. Integration, so we’ve had this conversation before. Integration, what tight and loose is, you know, getting away from the, getting away from this is a latency dependent definition in senses. And the reason that this also came up too is that kind of referring again to the Genesis Summit and conversations.

Room: Gina, when she was talking about integration across the Federation and the different systems and all, she was tight, they were talking about tight integration. They did not mean this in terms of latency, how we seem to be, how we’ve kind of gotten a little bit into our culture on that definition. they were really talking about literally, you know, the entire infrastructure coordinating very tightly on multiple different layers and how the user would actually be able to utilize this environment in a cohesive, friction-reduced sort of manner. And so kind of taking that, and if you go back and listen to her presentation, she actually has a couple of definitions, not just technical, but policy. elements, right? So kind of adopting that, I think would be kind of a good idea about what tight integration means, what integration means.

Room: And then we then we were determining, we kind of had a brief discussion about should we just throw away the term coupling? Should we really, you know, talk about integration and latency? These are the sort of things that we want to that we want to think about. So that was the first point. Struggle to distinguish between workflows. In general, we were talking about, again, applications, architectures, workflows. So hardware, software integration requirements for QEC resource estimates at a system level. Yeah, so I think the statement was, if you want to enable QEC research, instead of it being hard-coded on the QPU, can we, can a user run their own preferred QEC code? Can they allocate specialized hardware? Can I decode on my GPU instead of an FPGA that the QPU vendor provided? And then what kind of networking requirement?

Room: Because there is very strict requirements in terms of latencies when it comes to QEC codes and decoders. Right, that’s different from the application stuff. Yeah, and for the resource estimates part, we were looking at the telemetry discussion. So we were thinking about the QPU-based telemetry, which is what I think Michael was also talking about in the compilers, like how we can adapt that from QDMI maybe. Yeah, so we can, like some of the vendors actually provide calibration-based telemetry and we can use that for quantum error correction. Okay, goals and milestones connecting to the other working groups. We really want to, part of the conversation about workflows was that we were determining, you know, we had talked about like user personas, we had talked about end-to-end workflows.

Room: And we kind of came to the thought that The workflow, you know, like an end-to-end workflow for a particular application class that could show significant new results based on early fault-tolerant compute quantum computers. in the 2028-2029 time space. that’s going to have, that’s going to kind of intersect all of these components, all of our working groups. And there’s going to, and so if we’re thinking about this workflow component and how it affects like the hardware, the hardware placement, various, you know, all these different forms, this is kind of a very large, a big circle group effort.

Room: And so we were thinking that, you know, maybe one of the roles that we could take in our group is to kind of take charge or take ownership of the workflow that we all can that we figure out together what we’re going to try to use as our co-design example and then make sure that we’re constantly hitting. elements with the other groups. So in terms of the co-scheduling, in terms of elements of the compiler for this particular for this particular application and and and workflow, that’s one thing that we would be that we would be willing to do as our group.

Room: Well, in general, that’s what we were kind of thinking, and we’ll continue to, and then that would be to obviously to be, to gather requirements, to share requirements, and really, you know, we’ll be responsible for that step-by-step point by point, you know, for this particular application, for these user bases, figuring all that sort of stuff out. Oh, goals and milestones kind of with that. You know, we were also thinking that there were going to be user dependent, user persona dependent workflows. And we kind of were targeting on the quantum chemistry because the blueprint that came out was suggesting that in the next couple of years, early fault tolerant systems, we might get some successful results that would be significant enough to prove viability of the technology along the pathway. to look at those from different user persona definitions.

Room: Because what we were saying is that in the early phase of all of this, when we’re kind of trying to find an initial success in about two years time for early fault tolerant systems, that’s going to come from these power users that are going to be close to the hardware, that’s not going to need to have the abstraction layers. You know, we have to accept that that there’s going to be a certain stack that’s going to get them to that result. However, we need to show that we need to have that viable result, but then after that, we’re going to have a next wave of users right after that. They’re going to require abstraction. They’re going to require ecosystem.

Room: So basically thinking about, you know, a particular quantum chemistry workflow, how that might be implemented and interpreted and considered a success, by one power group users, like the first wave, how that might be considered viable, usable, a success from the second wave of folks, and then kind of what is that delta on development that we have to do. We thought that maybe this would also help give us a little bit of a logical sequence of action items of how we need to attack this. versus trying to solve everything at one time for all users.

## Resource Interface Report-Out

Room: In the morning, we discussed what’s available now because we have two groups. One is the QRMI community and the other is the QDMI community. at Munich, and they have been addressing this resource management interface issue for some time now. And then we also heard from Doug, who is busy pulling together QFW, which is Oak Ridge’s. framework for quantum, for their quantum HPC ecosystem. But it was time well spent, actually, because we did come to agreement on, for example, what is a quantum resource? We did come to agreement on what is a workload manager and what is a workflow. Let’s see, in the afternoon, we had discussions or input from stakeholders. The two sets of stakeholders were people who were developing workload managers, which was nice. I’d like to call out, thank Doug Jacobson for going over a new feature that’s in Slurm, HRES.

Room: And he gave us a nice sneak preview of how that will help us with quantum resource integration. And then Vanessa, who is with the Livermore Flux Team, which is a spiritual successor to Slurm, maybe, maybe that’s a good way to describe it. You know, they took, they did a clean slate design of a workload manager, and it’s a hierarchical scheduler, which is nice. So she was explaining how they’re doing. quantum resource integration. Then we also heard from Cameron, who is with, he’s actually the CTO over at Siemens for PBS. We also learned from the LSF, IBM LSF. Product manager, Gabor, and what they’re doing with respect to quantum integration.

Room: Okay, we heard from Michael from the operations team, which is great because while we have been doing operations, of course, at IBM and other QRMI community members have been… doing operations and LRZ of course is doing operations. So I did want to thank Amir and the rest of ORNL leadership we, yeah, yeah, sorry, Laura, I’m sorry. So, you know, coming from, I don’t know if you’ve ever visited the T.J. So we had the, of course, our overall goal is to come up with a standard community-driven interface, okay, for integrating quantum systems. So what we did want to try to establish is, well, what’s our methodology going to be? So we’re going to talk to the stakeholders, document their requirements. Then we’ll have the community of interface developers, you know, QMI, QDMI, you know, write the APIs to try to meet the requirements.

Room: And that’s where QFW and other efforts like that come in. What we did do today was we did create a list of milestones, publications, and artifacts. Okay, so this is our list of milestones. So this was actually my ploy to get fired. as the workstream organizer by putting so much on it, but surprisingly, it did not work. I don’t know if we’ll be able to get all of this done, but I’ll just try to step through some of the highlights. You know, the QDMI team. and the QMI team have been putting out papers. The first two papers up top describe the QDMI integration with AWS Bracket and as well as with IQM and Slurm. The QMI community just recently put out a paper that describes integrations with Slurm, PBS, LSF, Grid Engine, Kubernetes. Oh, Flux. Forgot Flux as well.

Room: We do want to establish a series of meetings because this one meeting was not enough. obviously. So we want to have our own work stream meetings, which of course will report up to open QSE in the open QSE meetings. We are making great progress, thanks to Doug, on the QFW test bed. We want to start using that to collect data about usage patterns. and hopefully benchmarking in the future. And then we’ll have some papers and eventually a specification on that. Now, the specification. We think we can pull one together, or at least a draft that we could distribute internally with an OpenQSE community, not declare it publicly, sometime early next year, okay, after some of all this experimentation, okay? And we would love to hear from the QEC teams, the operations teams about.

Room: We snuck in some milestones about evaluating HRES, so hopefully we can do that. You know, Slurm is the most popular workload manager right now. We do want to do a real quantitative analysis of QDMI and QMI. Right now, there’s a lot of qualitative analysis of the functionality. So, we want to spend time collecting data, understanding usage patterns, and do a real quantitative analysis to come up and, you know, that will support the draft specification, of course. Yeah, and then, Amir, I don’t know about this very last milestone. But I guess this depends on how many quantum systems that Oak Ridge buys, so we’re targeting 3, so Okay. So for the numeric and the terminology part, are we going to have like that part of the publication?

Room: I just, this may be a dumb question, but are there enough usage patterns on, for example, the Oak Ridge IQM system? Are there enough realistic users doing realistic things that you could use those usage patterns to evaluate? trade-offs in these interfaces? That’s certainly a concern we have, but it’s the only platform where we have a common back end. Systems, right? Are you tackling the problems of… actual device information formatting or anything like that or is that… One part of the community, that’s the QRM side, QRMI side, believes in flexibility and just allowing the hardware vendors to define it and pass that up. The other part of the community, QDMI, they’re trying to at least take a stab. at coming up with common formats and things like that.

## Software Architecture Report-Out

Room: All right, so we were in the software architecture working group today, and the morning session was around the runtime architecture. And basically we outlined that we wanted to create kind of an evidence-driven approach for how to formulate. this architecture specification, looking at some existing runnable prototypes and evaluating workflows across a kind of low latency to high latency, low HPC to high HPC, and try to learn from that what the standard should be. and understand some of these model boundaries, interfaces that need to be constructed to support this. The idea here is to build out these prototype inventory, looking at what implementations exist today, do some comparisons, and write a white paper around this, and compare and contrast those various interfaces to help inform our standard-based approach. We also want to test this on some real applications, so it’s application-formed, not application-specific.

Room: We want to make sure that we can run real applications through the workflows. So we went through one example application with VQE for molecular dynamics or quantum chemistry and looking at some of the pre and post processing tied to this application. AMD led a really nice discussion around the runtime layers that should exist and be able to be both application facing and targeting heterogeneous architectures and being able to identify ways in which to assign contracts across those different architectures or resources. We basically also said we really want to expose to different types of users, essentially the logical targets. That’s where people really, from the application level, would like to be able to get to. However, we really still need to provide access to physical details in the runtime environment in many cases.

Room: So there is kind of a desire to be in the logical space, but there’s also for performance purposes and practical nature, there’s still that physical element that’s required here as well. There was also some discussion around, for example, what should be part of the compiled object versus like a runtime parameters. So there’s definitely both information that we need to be interacting with the application layer as well as the compiler layer of stack to help inform some of these standard decisions.

Room: In terms of the milestones, we want to basically form a small working group in order to do this initial assessment across the different existing prototype and be able to do some kind of classification of those by layer and also set up these kind of experimental quadrants of how we want to assess each of these and be able to deploy some reference prototypes to run some workflows through those. And that will help us kind of outline what the specifications are that we’d like to try to converge to. We see this kind of as an iterative process and not something we want to just jump into directly. We would like to produce a white paper associated with this, with maybe some demos associated with it as well. I think we already kind of talked about this, Amir.

Room: So for the afternoon session, this was the control software architecture discussion led by Andrea. So the big picture is that we want to reframe the control stack as a software challenge. Rather than kind of hand adjusting all of the laboratory instruments, we want to have a fully software-defined stack driven by dynamic compilers and APIs and be able to apply a lot of intelligence at that control layer and data locality that allows us to provide smart control at that layer stack. So it’s not necessarily increasing the requirements of real-time communications across different parts of the stack. So being real clear about what we can do locally versus what we need to do in other parts of the stack. In terms of what we’re owning, it is the physical and software interfaces, as well as the fast feedback protocols and analysis of the scaling data bottlenecks.

Room: And we need to hand off to the compiler group the intermediate representations. We’re looking at building a hardware-software boundary specification with the fast feedback protocol definition and a reference proof of concept for a real hardware test bed. We’d love to get that real hardware test bed deployed here or in combination with some of the other DOE science labs. Three of the big questions we’re tackling next is what do we draw in terms of an exact line between the pulse level access and logical QEC primitives? What tasks must happen on chip versus asynchronously in software? And how do we build open vendor neutral contracts so that no one gets locked into a proprietary hardware? We have 4 milestones. The top two, I think we’re going to try to target first. And this is basically doing an analysis of what exists in this space.

Room: And outlining some of the strict latency bandwidth and other requirements around this integration, and then defining the hardware software boundaries. We then want to be able to use this information to help inform our fast feedback protocol definition, and finally a reference implementation. We also are looking to build a Slack channel, have some regular meetings focused on milestones one and two. And we need to have outreach activities to recruit an army of grad students and other metalheads wanting to get involved in this and do some milestone planning and timelines for the subtasks.